AT-A-PSL1

PoE сплиттер

- это 100-метровый разветвитель PoE на большие расстояния, независимо разработанный компанией Fengrunda. При использовании с PoE-коммутатором Fengrunda он может передавать данные на сверхдальние расстояния до 500 метров (100 Мбит/ с) и 800 метров (10 Мбит/ с); Один порт передачи данных RJ45 (PD) поддерживает стандартный ввод IEEE802.3af; другой конец имеет порт передачи данных RJ45, источник постоянного тока. порт вывода питания (12 В/ 1 А); без настройки, подключи и играй.

Product Features

- Support IEEE 802.3 af power receiving standard

- Support 12V/1A DC output

- Support 10/100Mbps data transmission, can switch rate mode independently

- 100Mbps data transmission distance up to 500 meters (need to be used with Fengrunda long-distance PoE switch)

- 10 Mbps data transmission distance up to 800 meters (need to be used with Fengrunda long-distance PoE switch)

- Surge: Common mode 6KV for network port; ESD: 8KV for air, 6KV for contact

- No need to configure, plug and play

Product specifications

| Model | PD802GH |
| --- | --- |
| Protocol Standard | IEEE802.3, IEEE802.3u, IEEE802.3x, IEEE802.3af |
| Interface Type | One LAN IN RJ45 port (PD) One LAN OUT RJ45 interface A DC power output interface: (outer diameter * inner diameter) 5.5*2.1mm |
| Network medium | Standard Cat5e and above twisted pair |
| Transmission distance | 100Mbps data transmission distance up to 500 meters 10Mbps data transmission distance up to 800 meters (Need to be used with Fengrunda long-distance PoE switch) |
| PoE Input | IEEE802.3af |
| Power Output | DC output: 12V/1A(Max) |
| LED Indicator light | Red light: Power indicator Green light: 100Mbps rate indicator Yellow light: 10Mbps rate indicator |
| Physical specifications | Dimensions (length x width x height): 98.7*60*26 mm Weight: 0.2kg |
| Operating temperature | -10~50℃ |
| Storage temperature | -40~70℃ |
| Working humidity | 10%~90% No condensation |
| Storage humidity | 5%~90% No condensation |
| Protection level | Surge: The common mode of the network port is 6KV Electrostatic discharge (ESD): Air 8KV, contact 6KV |
| Safety Regulations | CE/FCC/CCC/RoHS |

Schematic diagram